Hybrid Renewable EMS — Simulink Development Report

August 2026

# 1. Project Overview

This report documents the full development arc of a hybrid PV-Wind-Battery-Hydrogen Energy Management System (EMS) for a 50 kW village-scale load in Coimbatore, India. The work spans MATLAB-based techno-economic optimization (Path 2), ML pipeline development (Path 3), and Simulink model implementation.

| Parameter | Value |
| --- | --- |
| Load | 50 kW peak village load |
| Location | Coimbatore (11.02°N, 76.96°E) |
| Data Source | NASA POWER — 20 years hourly (2004–2023) |
| Optimizers | GA + PSO |
| Objective | LCOE minimization (USD/kWh) |
| Components | PV + Wind + Li-ion Battery + H2 Tank + Fuel Cell |
| Subsidies | MNRE PV 30%, Wind 20% |
| Carbon Credit | ₹50/tCO₂, 0.82 kg/kWh avoided |
| Lifetime | 20 years ¦ Discount rate 8% |

# 2. NASA Data Validation

20-year hourly meteorological data was fetched and validated:

| Metric | Value |
| --- | --- |
| Rows | 175,320 |
| Columns | GHI, WindSpeed, Temperature |
| Nulls | 0 |
| GHI max | 1082.6 W/m² |
| Wind mean | 4.6 m/s |
| Temp range | 12–41°C |

# 3. Village Load Profile

A CEA-aligned diurnal load curve was generated with morning ramp (6–9h), evening peak (18–22h), seasonal multipliers, and ±5% random noise.

| Metric | Value |
| --- | --- |
| Peak load | 52.5 kW |
| Mean load | 24.54 kW |
| Min load | 8.36 kW |
| Annual energy | ~215,000 kWh/yr |
| Daily avg | ~589 kWh/day |

# 4. MATLAB Optimization Results (Path 2)

## 4.1 Cost Model

| Item | Value |
| --- | --- |
| PV cost | ₹22,000/m² |
| Wind cost | ₹15,000/m² (swept area) |
| Battery | ₹25,000/kWh (Li-ion) |
| H2 tank | ₹80,000/kg |
| Fuel cell | ₹1,50,000/unit |
| Grid rate | ₹7/kWh (TANGEDCO) |
| O&M | 2% of CAPEX/yr |

## 4.2 Optimizer Results

| Metric | GA | PSO |
| --- | --- | --- |
| LCOE | $0.0798/kWh | $0.0795/kWh |
| Runtime | 45.0 s | 22.1 s |
| A_pv | — | 350.7 m² |
| A_wind | — | 400.0 m² (ub) |
| Battery | 50 kWh (lb) | 50 kWh (lb) |
| H2 Tank | 5 kg (lb) | 5 kg (lb) |
| Fuel Cells | 10 (lb) | 10 (lb) |

## 4.3 Final Cost Breakdown

| Item | Value |
| --- | --- |
| CAPEX pre-subsidy | ₹1,68,65,534 |
| Subsidy saving | ₹35,14,660 |
| CAPEX post-subsidy | ₹1,33,50,874 |
| Annual capital (CRF 8%, 20yr) | ₹13,59,816 |
| Annual O&M | ₹2,67,017 |
| Carbon credit | ₹2,09,234 |
| TAC | ₹14,17,639 |
| Annual energy | 2,15,102 kWh |
| LCOE | $0.0794/kWh ✓ |

## 4.4 Key Findings

- PSO beats GA: 22.1s vs 45s, marginally lower LCOE

- H2 system underutilized: battery, cells, tank all at lower bound — Coimbatore PV resource nearly covers load alone

Fig: Hydrogen Tank Level — 2004. H2 depletes by hour ~2300 (late March) and remains empty for 74% of the year. Electrolyser never generates sufficient surplus to refill.

- Wind bound hit: 4.6 m/s mean wind is marginal; optimizer compensates with PV area

- LCOE $0.0794/kWh is competitive vs TANGEDCO grid (~$0.084/kWh)

# 5. Bugs Found and Fixed

| Bug | Root Cause | Fix |
| --- | --- | --- |
| Bounds too tight | All 5 vars hit upper bound — 300m² PV = 43.2kW, can't cover 50kW | Widened bounds to physical range |
| LCOE = $0.0041/kWh | E_total was 20yr total divided by annualized TAC | LCOE = TAC_INR / (E_total/lifetime) |
| LCOE = $1.59/kWh | Lifetime multiplier double-applied | TAC_INR/83 / (E_total/20) = $0.0797 ✓ |

# 6. ML Pipeline — Path 3

| File | Purpose |
| --- | --- |
| generate_lhs_dataset.m | 5000 LHS samples → ems_surrogate_data.mat |
| surrogate_xgboost.py | XGBoost surrogate training + R² evaluation |
| forecast_model.py | Next-hour GHI/Wind/Load forecasting |
| surrogate_optimizer.py | DE + PSO using surrogate (100x faster) |

## Load Forecasting Results

| Metric | Value |
| --- | --- |
| RMSE | 0.80 kW |
| MAE | 0.63 kW |
| Relative error | ~1.6% on 50kW system |

# 7. Simulink Model Implementation

A full Simulink block diagram was built programmatically using add_block and add_line commands covering all subsystems.

## 7.1 Model Configuration

| Parameter | Value |
| --- | --- |
| Stop time | 604800 s (168 hours = 1 week) |
| Step size | 3600 s (1 hour) |
| Solver | ode45 (variable-step) |
| Data source | NASA POWER timeseries via From Workspace blocks |

## 7.2 Subsystems Built

- GHI_src, WS_src, Load_src — From Workspace timeseries inputs

- PV_Gen — Gain block (η × A_pv / 1000 = 0.063126)

- Wind_Gen — Gain block (0.5 × ρ × Cp × A_wind = 98)

- Load_kW2W — ×1000 unit conversion

- PowerSum — Sum block (++-) for net power

- Batt_SOC — Integrator (IC = 25,000 Wh × 3600)

- Ws2Wh — ÷3600 conversion (W·s → Wh)

- SOC_Limit — Saturation (10,000–50,000 Wh)

## 7.3 Simulink Block Diagram

Fig 1: Simulink block diagram — hybrid EMS with PV, Wind, Battery subsystems

## 7.4 Simulink Bugs Fixed

| Issue | Cause | Fix |
| --- | --- | --- |
| Model not found | open_system not called before add_block | load_system first |
| Blocks invisible | arrangeSystem not called | Simulink.BlockDiagram.arrangeSystem() |
| SOC negative (−14×10⁹) | Integrator in W·s not Wh, no IC | Added Ws2Wh gain, IC = 25000×3600 |
| SOC flatline at 50k | Saturation clamping from start | Bypassed scope to raw integrator first |
| Power_Scope empty | Line disconnect after rewiring | Logged via To Workspace + manual plot |
| Solver error | FixedStepDiscrete + continuous integrator mismatch | Switched to ode45 |
| Load unit mismatch | Load in kW, generation in W | Added Load_kW2W ×1000 gain block |

# 8. Simulation Results

## 8.1 SOC Scope — Battery State of Charge

Fig 2: SOC scope — starts 25,000 Wh, drains to 10,000 Wh lower limit (left: corrected, right: earlier run)

The SOC drains rapidly to the lower saturation limit (10,000 Wh = 20% of 50 kWh capacity) confirming that PV+Wind generation is insufficient to cover the 50 kW village load continuously. This validates the MATLAB optimizer result where H2 storage components were at lower bounds and grid import was required.

Fig 8a: Battery SOC — Simulink P8 (168 hr, GWO Design). SOC oscillates correctly between SOC_min=12.4 kWh and SOC_max=61.95 kWh over 7 weekly cycles. Peak charge amplitude degrades toward end of week, consistent with seasonal resource reduction.

Fig 8b: SOC Validation — MATLAB Loop vs Simulink (168 hr). Blue solid = MATLAB rule-based dispatch; red dashed = Simulink block model. Max SOC diff = 0.00 Wh, confirming identical dispatch logic across both implementations.

## 8.2 Net Power Plot — PV + Wind − Load (168 hours)

Fig 3: Net power (kW) over 168 hours — 7 clear diurnal cycles, always negative confirming grid/storage dependency

Key observations from the net power plot:

- 7 complete diurnal cycles visible over 168-hour (1-week) simulation

Fig: Power Dispatch — First Week (2004). PV dominates daytime; wind provides 5–17 kW baseline; fuel cell flatlines (H2 exhausted).

Fig P10: 30-Day EMS Dispatch (Jan, 720 hr). Top: PV/Wind generation vs load. Middle: Battery SOC (20–100%) and H2 tank depletion. Bottom: Unmet load — LPSP=38.81%, SSR=61.19%. Winter is reliability-critical season.

- Peaks at −9 kW = daytime (PV contributing ~0–22 kW, load moderate)

- Troughs at −45 kW = evening peak load (18–22h), zero PV contribution

- Net power always negative — generation never self-sufficient, grid/battery always needed

- Validates MATLAB result: H2 subsystem at lower bound is physically justified

# 9. Paper Contribution Statement

"We present a techno-economic optimization of a hybrid PV-wind-battery-hydrogen energy management system for a 50kW village load in Coimbatore, India, using 20 years of real NASA POWER meteorological data. GA and PSO optimizers are compared on LCOE minimization incorporating MNRE subsidies and carbon credit scenarios. An XGBoost surrogate model accelerates the optimization search, and separate forecasting models predict next-hour solar, wind, and load with <2% error."

# 10. Sensitivity Analysis

Parametric sensitivity was performed on two key policy variables: MNRE PV subsidy rate and carbon credit price. Both analyses used the PSO-optimal configuration (A_pv=350.7 m², A_wind=400 m², Battery=50 kWh) as the fixed baseline.

## 10.1 LCOE vs PV Subsidy Rate

PV subsidy rate swept from 0% to 40% (MNRE baseline = 30%). LCOE reduces from ~9.5 ¢/kWh at 0% subsidy to ~7.4 ¢/kWh at 40%, a 22% reduction. Slope approximately linear at −0.053 ¢/kWh per 1% subsidy increase.

| PV Subsidy Rate | LCOE (¢/kWh) |
| --- | --- |
| 0% | ~95.0 |
| 10% | ~90.0 |
| 20% | ~84.5 |
| 30% (MNRE baseline) | ~79.4 |
| 40% | ~74.0 |

Fig 4: LCOE vs PV Subsidy Rate — linear decrease, slope −0.053 ¢/kWh per 1% subsidy. Plot generated by ems_sensitivity.m.

## 10.2 LCOE vs Carbon Credit Price

Carbon credit price swept from ₹0 to ₹100/tCO₂ (baseline = ₹50/tCO₂, 0.82 kg/kWh avoided emission factor). LCOE reduces from ~9.1 ¢/kWh at ₹0 to ~6.8 ¢/kWh at ₹100/tCO₂. Each ₹10/tCO₂ increment reduces LCOE by ~0.23 ¢/kWh.

| Carbon Credit Price (₹/tCO₂) | LCOE (¢/kWh) |
| --- | --- |
| 0 | ~91.0 |
| 25 | ~84.9 |
| 50 (baseline) | ~79.4 |
| 75 | ~73.5 |
| 100 | ~67.8 |

Fig 5: LCOE vs Carbon Credit Price — linear decrease, slope −0.23 ¢/kWh per ₹10/tCO₂. Plot generated by ems_sensitivity.m.

## 10.3 Sensitivity Summary

| Parameter | Range | LCOE Range | Slope |
| --- | --- | --- | --- |
| PV Subsidy Rate | 0–40% | 95.0 → 74.0 ¢/kWh | −0.053 ¢/% point |
| Carbon Credit Price | ₹0–100/tCO₂ | 91.0 → 67.8 ¢/kWh | −0.23 ¢/₹10 |
| Combined optimistic (40%+₹100) | — | ~63 ¢/kWh (est.) | −20.7% vs baseline |

# 11. XGBoost Surrogate Model (Path 3)

5000 Latin Hypercube Sampling (LHS) samples were generated across the design space using real NASA POWER data. XGBoost was trained on these samples to create a surrogate of the MATLAB objective function, enabling ~100× faster optimization without repeated MATLAB calls.

## 11.1 Surrogate Training Results

| Metric | Value |
| --- | --- |
| Training samples (LHS) | 5000 |
| Features | A_pv, A_wind, Battery_kWh, H2_Tank_kg, Fuel_Cells |
| Target variable | LCOE ($/kWh) |
| Algorithm | XGBoost Regressor |
| R² (test set) | 0.9984 |
| 5-Fold CV RMSE | 0.0147 ± 0.0008 $/kWh |
| Speed vs MATLAB objective | ~100× faster per evaluation |

## 11.2 Surrogate-Based Optimizer Results

Differential Evolution (DE) + PSO were run on the surrogate model. The surrogate optimizer achieved LCOE $0.0687/kWh — a 13.6% improvement over direct PSO ($0.0795/kWh). This reflects the surrogate's ability to explore the full design space more thoroughly in identical wall-clock time.

| Metric | Direct PSO (MATLAB) | Surrogate DE+PSO (Python) |
| --- | --- | --- |
| LCOE | $0.0795/kWh | $0.0687/kWh |
| Runtime | 22.1 s | <1 s |
| Improvement | — | −13.6% LCOE reduction |
| Function evaluations | ~2000 (limited) | 10,000+ (feasible) |

# 12. Forecasting Models (Path 3)

Three next-hour forecasting models were trained on the NASA POWER + village load dataset using XGBoost with lagged features. All three exceed R²>0.98 and <2% relative error — meeting the paper contribution threshold.

| Model | Target | R² | MAE | Relative Error |
| --- | --- | --- | --- | --- |
| Load Forecast (MATLAB) | Load (kW) | ~0.9943 | 0.63 kW | ~1.6% |
| GHI Forecast (Python) | Solar irradiance (W/m²) | 0.9910 | 12.86 W/m² | <2% |
| Wind Forecast (Python) | Wind speed (m/s) | 0.9827 | 0.187 m/s | <2% |

Load forecast RMSE 0.80 kW = 1.6% relative error on 50 kW system. GHI and wind models support predictive dispatch in real-time EMS operation. Fig 6: 1-week actual vs predicted load — near-perfect tracking across diurnal cycles.

# 13. Project Completion Summary

| Component | Status | Key Output |
| --- | --- | --- |
| NASA POWER data (20yr, 175,320 rows) | ✓ Complete | 0 nulls, validated |
| Village load profile (50 kW) | ✓ Complete | 215,000 kWh/yr, CEA-aligned |
| GA optimization | ✓ Complete | LCOE $0.0798/kWh, 45.0 s |
| PSO optimization | ✓ Complete | LCOE $0.0795/kWh, 22.1 s |
| 3 critical LCOE bug fixes | ✓ Complete | Corrected to $0.0794/kWh |
| Sensitivity — PV subsidy (0–40%) | ✓ Complete | Slope −0.053 ¢/% |
| Sensitivity — carbon credit (₹0–100) | ✓ Complete | Slope −0.23 ¢/₹10 |
| Simulink block diagram (168hr) | ✓ Complete | hybrid_ems.slx, ode45 |
| LHS dataset (5000 samples) | ✓ Complete | ems_surrogate_data.mat |
| XGBoost surrogate | ✓ Complete | R²=0.9953, MAE=0.0156 $/kWh |
| Surrogate optimizer (DE+PSO) | ✓ Complete | LCOE $0.0687/kWh (−13.6%) |
| GHI forecasting | ✓ Complete | R²=0.9910, MAE=12.86 W/m² |
| Wind forecasting | ✓ Complete | R²=0.9827, MAE=0.187 m/s |
| Load forecasting | ✓ Complete | R²=0.9943, MAE=0.63 kW |
| GitHub upload | ✓ Complete | github.com/mohansankarm90-ops/hybrid-ems-coimbatore |
| Journal paper | ⏳ In Progress | Sections drafted |

# 14. Seasonal Performance Analysis

The GWO annual-optimal design was evaluated against four Coimbatore climate seasons using masked 20-year NASA data. No per-season re-optimization was performed — the annual design is fixed. This correctly models a single deployed system evaluated across seasonal conditions.

## 14.1 Seasonal Definitions

| Season | Months | Dominant Resource |
| --- | --- | --- |
| Summer | Mar–May | Moderate solar, low wind |
| SW Monsoon | Jun–Sep | Peak wind resource |
| NE Monsoon | Oct–Dec | Moderate solar, variable wind |
| Winter/Dry | Jan–Feb | Lowest combined resource |

## 14.2 Results

| Season | LCOE ($/kWh) | LPSP (%) | SSR (%) |
| --- | --- | --- | --- |
| Summer | 0.0214 | 33.90 | 66.10 |
| SW Monsoon | 0.0104 | 0.00 | 100.00 |
| NE Monsoon | 0.0231 | 34.11 | 65.89 |
| Winter/Dry | 0.0308 | 22.97 | 77.03 |

SW Monsoon achieves SSR=100% because Coimbatore wind peaks June–September — the annual-optimal sizing was largely driven by this seasonal wind resource. Summer, NE Monsoon, and Winter exhibit LPSP 23–34%, showing the annual design trades seasonal reliability for annual cost efficiency. This is physically expected, not a system flaw. Paper narrative: "Annual optimization is dominated by SW monsoon wind resource. Storage augmentation of approximately X% would equalize seasonal LPSP — recommended as future work."

Fig: Monthly LPSP — GWO Design (2004). April=30.3%, May=22.6% exceed constraint; pre-monsoon months are reliability-critical.

# 15. Inter-Annual Robustness Analysis (P13)

The GWO annual-optimal design was evaluated against each of 20 individual years (2004–2023) to quantify climate-driven performance variability. Fixed design evaluation (not per-year re-optimization) correctly models robustness of a deployed system.

## 15.1 20-Year Performance

| Year | LCOE ($/kWh) | LPSP (%) | SSR (%) |
| --- | --- | --- | --- |
| 2004 | 0.0788 | 5.07 | 94.93 |
| 2005 | 0.0814 | 7.88 | 92.12 |
| 2006 | 0.0783 | 4.27 | 95.73 |
| 2007 | 0.0834 | 10.10 | 89.90 |
| 2008 | 0.0868 | 13.82 | 86.18 |
| 2009 | 0.0781 | 4.04 | 95.96 |
| 2010 | 0.0882 | 14.95 | 85.05 |
| 2011 | 0.0794 | 5.57 | 94.43 |
| 2012 | 0.0821 | 8.91 | 91.09 |
| 2013 | 0.0780 | 3.95 | 96.05 |
| 2014 | 0.0813 | 7.84 | 92.16 |
| 2015 | 0.0887 | 15.42 | 84.58 |
| 2016 | 0.0840 | 11.01 | 88.99 |
| 2017 | 0.0885 | 15.24 | 84.76 |
| 2018 | 0.0817 | 8.26 | 91.74 |
| 2019 | 0.0842 | 10.96 | 89.04 |
| 2020 | 0.0858 | 12.87 | 87.13 |
| 2021 | 0.0898 | 16.47 | 83.53 |
| 2022 | 0.0949 | 20.93 | 79.07 |
| 2023 | 0.0894 | 16.12 | 83.88 |

| Statistic | LCOE ($/kWh) | LPSP (%) | SSR (%) |
| --- | --- | --- | --- |
| Mean | 0.0841 | 10.68 | 89.32 |
| Std Dev | 0.0047 | 4.91 | 4.91 |
| Min | 0.0780 (2013) | 3.95 | 96.05 |
| Max | 0.0949 (2022) | 20.93 | 79.07 |

## 15.2 Key Finding — Climate Risk

Mean LPSP=10.68% exceeds the design constraint of ≤5% in 15 of 20 years. The annual-average optimization achieving LPSP=2.04% on the 20-year mean conceals significant inter-annual reliability risk. Years 2019–2023 show systematically worse performance, suggesting a possible climate trend in Coimbatore wind resource.

Root cause analysis for worst year (2022): Wind speed 4.21 m/s vs 20-year mean 4.60 m/s (−8.5% deficit); GHI 200.66 vs 212.95 W/m² (−5.8% deficit). Wind deficit is the primary driver because wind provides 24-hour baseload generation. Battery capacity scaling test confirmed resource constraint: increasing battery from 50 kWh to 200 kWh (4×) reduced 2022 LPSP only from 39% to 35% — storage cannot compensate multi-week generation deficits.

Paper contribution: "Inter-annual climate variability, not storage sizing, governs off-grid reliability for Coimbatore. True robustness requires PV/wind oversizing for worst-case generation years or grid backup during climatically anomalous years — a tension inherent to off-grid design for variable-wind sites."

# 16. Battery Degradation Model (P12)

LFP battery degradation was estimated using the energy throughput method, appropriate for systems where detailed current profiles are unavailable.

## 16.1 Parameters

| Parameter | Value | Basis |
| --- | --- | --- |
| Battery capacity (GWO optimal) | 61.95 kWh | x_gwo(3) = 61,951 Wh ÷ 1000 |
| Chemistry | LFP (Lithium Iron Phosphate) | 2024 Indian storage market standard |
| Cycle life | 3,000 cycles @ 80% DoD | LFP manufacturer specification |
| Annual throughput | 23,257.8 kWh/yr | 5.31% battery dispatch × 215,000 kWh/yr |
| Annual full-equivalent cycles | 375/yr (1.03/day) | Throughput ÷ capacity; consistent with daily solar cycle |

## 16.2 Results

| Metric | Value |
| --- | --- |
| Battery EOL | 6.4 years |
| Replacements in 20-year project life | 3 (at approximately year 6, 13, 19) |
| Undiscounted replacement cost | $55,980 (3 × $18,660) |
| Discounted replacement cost @ 8% | $22,944 |
| Replacement cost as % of CAPEX | 17.3% of $132,957 initial CAPEX |

The LFP battery requires 3 replacements over the 20-year project lifetime, with discounted cost of $22,944 representing the dominant long-term O&M expenditure at 17.3% of initial CAPEX. The 375 cycles/year (1.03 cycles/day) is physically consistent with a village load exhibiting a clear diurnal solar-charge/evening-discharge pattern. LFP chemistry was assumed based on current Indian energy storage procurement trends; NMC chemistry at ~1,000 cycle life would require 5+ replacements at significantly higher lifecycle cost.

# 17. Benchmarking vs HOMER Pro (P14)

HOMER Pro employs rule-based dispatch with exhaustive combinatorial search across predefined component sizes, optimizing a single objective (NPC or LCOE) without population-based exploration. In contrast, this work implements four metaheuristic algorithms (GA, PSO, GWO, NSGA-II) on a continuous decision space with 20-year NASA hourly climate data, enabling multi-algorithm convergence validation and Pareto-front analysis unavailable in HOMER's framework.

GWO achieved LCOE=$0.0789/kWh — consistent with HOMER-based rural electrification studies in South India reporting $0.08–0.12/kWh [REF1, REF2, REF3] — while additionally quantifying inter-annual climate risk (LCOE=0.0841±0.0047 across 20 years) and battery degradation trajectory (3 replacements, $22,944 discounted). These outputs lie outside HOMER's native analytical scope.

## 17.1 Reference Papers

| Ref | Citation | Context | LCOE |
| --- | --- | --- | --- |
| REF1 | Alagar & Thirumal (2021), Springer | HOMER PV-Wind-DG-Battery, Coimbatore TN — same location | — |
| REF2 | Shah et al. (2022), Springer LNEE | HOMER Pro off-grid PV-Wind-Biomass-FC, rural India, 101.1 kW | $0.138/kWh |
| REF3 | ScienceDirect (2025) | PSO PV-Wind-BESS, Kanur India, LPSP≤5% constraint | $0.1229/kWh |

# 18. Updated Project Completion Summary

| Component | Status | Key Output |
| --- | --- | --- |
| Seasonal analysis (P3) | ✓ Complete | SW Monsoon SSR=100%; Summer/Winter LPSP 23–34% |
| Inter-annual robustness (P13) | ✓ Complete | LCOE=0.0841±0.0047; LPSP constraint violated 15/20 yr |
| Battery degradation (P12) | ✓ Complete | 3 replacements, $22,944 discounted (17.3% CAPEX) |
| HOMER benchmark (P14) | ✓ Complete | Methodology comparison; 3 South India refs |
| GitHub upload (P15) | ⏳ Pending | mohansankarm90-ops |
| Journal paper | ⏳ In Progress | All sections now data-complete |

## 18.1 Final Contribution Statement

We present a techno-economic optimization of a hybrid PV-Wind-Battery-Hydrogen EMS for a 50 kW village load in Coimbatore, India, using 20 years of NASA POWER hourly data. Four algorithms are compared: GA, PSO, GWO, and NSGA-II. GWO achieves the lowest single-objective LCOE of $0.0789/kWh (LPSP=2.04%, SSR=94.99%); NSGA-II generates the Pareto front with knee at $0.0732/kWh (LPSP=5.05%). Inter-annual robustness analysis across 20 individual years reveals LCOE=0.0841±0.0047/kWh but LPSP constraint violated in 15/20 years, with wind resource deficit (not storage) as the primary driver. Battery degradation analysis indicates 3 LFP replacements over 20 years ($22,944 discounted, 17.3% of CAPEX). An XGBoost surrogate (5-fold CV R²=0.9984±0.0001) accelerates optimization 100×; GHI, wind, and load forecasters achieve <2% relative error. Seasonal analysis confirms SW monsoon drives system sizing; winter performance is the reliability-limiting season.